Standard Operating Procedure (SOP)

| Name of the Factory |  |
| --- | --- |
| Address |  |
| Title Of the SOP | Monitoring chemical storage requirements of different buyers/brands required for operation. |
| Purpose | To ensure proper chemical Storage in the facility. |
| Objectives | To ensure proper monitoring of all applicable new or changing compliance requirements regarding chemical Storage. |
| Organization /Monitoring Body |  |
| Communication |  |
| Procedures | - All chemical store in good ventilation to keep humidity and temperature at optimal levels. No direct sunlight on chemicals. - All chemicals stored according to SDS information. - All SDS post in local language available in storage areas. - Clear instruction and safety precaution (in local languages) on chemical handling clearly stated in the working area. - “Chemical Storage Area “, always visible in all storage areas. - Chemicals stored on impervious surfaces in a systematic way and easily identified by labels. - All chemicals stored always recorded in Chemical Inventory list (CIL) to avoid excessive storage of dangerous chemicals. The CIL should be updated and reviewed regularly - Incompatible chemicals is always stored separately. - Chemicals used according to first-in-first-out basis. - Chemicals must be sealed when not in use. - Food, Beverages and Tobacco is not be allowed in chemical storage area. - First aid box, Personal Protective Equipment (PPE) box, Proper extinguisher, emergency shower/eye wash station, spill kits, Secondary containment is always placed in chemical storage area. - Chemical storage room is not allowed to enter without permission. |
| . |  |
| Validity of SOP | Issued on Effective from Revised date 01.01.2022 01.01.2022 01.01.2023 |
| SOP approved by |  |